The Log home

What is a Log home?

By definitions, logs are the single most predominant characteristics of these homes.  The very reason builders and owners alike, are drawn to this form of construction.

What could be more natural?  The appeal of logs is apparent to all log builders and owners alike, visitors to log structures are immediately drawn to touch the logs.  (How often do you see people caress Drywall?)

When you close your eyes and think of a log cabin or home what do you see?

The rustic home of early settlers?

Or an oasis of wood?

Both are examples of what you can find along with everything in between.  The longevity and strength of log homes is evidenced in the well-built log structures of Norway and Russia.  Some of which are still standing and used after more than 800 years.  The integrity of the structural design is essential to a long – lived legacy and a look a structural component is critical.  Points of consideration:

- Choice of what logs to use. – understanding the differences and characteristics of various species.

25 min

- Building styles – home, cabin, summer cottage.

25 min

- Who is building

15 min

Choice of logs to use:

there are over seven hundred species of trees in the USA, however, manufacturers only use roughly two dozen of them to build log homes. So, how do you know which species to opt for if you do need to source logs?

Choosing the right logs is an important decision that needs to be made at the beginning of your planning process. The logs will determine the overall look and structure of your finished log home, the cost, and how well your home with withstand Mother Nature.

Deciding as to which species of tree you should use to build your log home will fall to several key things; appearance, cost, R-value, decay resistance, sturdiness/stability, and availability.

Let’s take a look at each of these factors:

## Appearance

The look of the logs, and what you find attractive is very much dependent on your personal taste, but you must also bear in mind that the appearance of logs can change when you apply preservatives or through weathering.

If you are selecting your own logs, you will want to make sure they are straight and have a good length of around 10-12m.

## Cost

The cost of logs can vary quite largely depending on the species.

Usually, slow-growing species such as northern white cedar are more expensive and fast-growing species such as yellow pine are less expensive.

The price can also be affected by the area in which you live in. If a species of wood is not readily available in your location, you will have the added cost of transportation.

## R-Value

Insulation efficiency varies in different species of logs; however, this is only very marginal and there are lots of other ways to insulate your log cabin and make it as energy efficient as possible, therefore the species you choose won’t have a huge impact.

## Decay resistance

Trees produce their own natural decay resistance though toxins. These toxins help the tree to resist attack from insect infestations and fungi.

The older the tree, the higher the concentration of toxins, so therefore the more heartwood a tree has, the more resistant it is to decay.

Fast-growing species have more sapwood, so are typically less resistant to decay but should not be ruled out as they are easy to protect and maintain with wood preservatives.

Even the most naturally highly resistant species such as cypress, redwood, and western red cedar, will still require treatment and maintenance.

## Stability

Almost all logs are likely to shrink and settle. Other concerns that you may have is that they will twist, warp and check.

You can avoid these concerns becoming a reality and ensure your log cabin is stable and sturdy by choosing appropriate good-quality logs during the selection stage.

## Availability

Your choice of log will very much depend on what is available in your local area unless you are willing to pay a premium for transportation costs. Once you have chosen the species of logs you are going to use, you will need to consider where the trees have been grown, and how they are cut and dried.

## Where are the trees grown?

The higher the altitude the trees are grown in, the better.

Slow-growing trees produce much more dense wood and have tight growth rings which usually yield fewer cracks as the logs are drying.

Many trees are now grown in tree farms, to keep up with the demand for trees needed. This results in less resistant trees as they have not had the time to develop as many concentrated toxins as a tree grown over a longer period of time.

Trees that are grown quickly are more likely to be mostly sapwood too, which will lead to a log cabin which is prone to shrink and warp more substantially.

## When and how the wood is cut?

Logs should be felled in the winter; to make sure the sap content is at is lowest.

If you are buying logs from a manufacturer, it is important to also ask them which cut off the log they use.

Although more expensive, you ideally want it to be mostly heartwood, the strongest part of the tree, and not the sides of the log.

## How the logs are dried?

The logs in log houses have a varying degree of moisture content; all logs have moisture when freshly cut. In the case of handcrafted logs moisture will naturally leave the timber, drying it out until it stabilizes with its climate. This drying-out causes movement and shrinking of the log's diameter. As logs and timbers dry, the differential shrinkage (radial versus tangential) causes small cracks (known as "checks") to open slowly over time. Checking is a natural process in both air- and kiln-dried logs. This occurs in all log houses regardless of construction method or how the timber is allowed to dry and is considered normal.

Three main types of drying:

Green Drying – After construction green logs dry in service in use of the log building.  Within about four years, green logs reach equilibrium with local conditions and have moisture content between 6% and 12% depending on Local climate, season and location

Air Dried Logs – Allow fresh cut logs to sit outside in the open air to dry naturally.  The timbers are stacked with Spacers (known as “stickers”) between them.  This process allows the moisture to fall as the timber dries.  A log with a diameter of 8 inches will usually reach equilibrium in about 18 months.  Air circulation is critical, or the logs may begin to rot before properly drying especially in humid regions.  The logs must be kept under some type of cover to reduce the impact of rain and snow elements on the drying process.

Kiln dried logs – Green timber is placed inside a large oven where the moisture is removed from them.  These logs can suffer severe checking and cracking if the kiln controls are not properly maintained and monitored.  Use of a kiln can reduce the drying time from many months to several weeks. Kiln dried logs can be expected to shrink and settle over time, but to a lesser extent than green logs.

## Log Types

The most popular logs used to build log home are very much dependent on the trees grown in each area, or the availability and cost of trees in a particular area. These are the most common trees used:

America
Pine (white, yellow, and red) Eastern or Western white Cedar, Cypress, Spruce, Fir, Hemlock, Ponderosa Pine, Lodgepole Pine, and hardwoods such as Oak, Poplar, and Walnut.

Europe
Spruce, Pine

Australia
Douglas fir, Cypress, Larch, Redwood

Logs are a renewable resource.  Wise and selective use of our trees and our forestry practices must be seriously re-evaluated.  Clear-cut practices, reforestation, and selective logging are all influencing what logs are available.  The log builder can make efficient use of our forest heritage. Building with logs can make sense.  What it’s built with and how ensure it’s most efficient and effective use as a priority.

Let’s take a better look at each species of tree. It is difficult to call which one species is the best, choosing your logs is very particular to the individual and the area in which you are intending to build. Hopefully, the following breakdown will help you to think about the most suitable logs for you to use:

You should now be in a better position to see which type of wood species you prefer based on:

- Appearance

- Cost

- Insulation (R-Value)

- Decay resistance

- Stability

- Availability in your location.

Regardless of which logs you end up choosing to build your log cabin home, you need to ensure that your cabin is well designed during the planning process to allow for shrinkage, that your logs are treated appropriately before construction and that you continue to maintain the logs well upon completion.

Video

Episode #1 Log Home Construction - Stacking Log Walls, Framing & Building Techniques - YouTube

Time 11:23

## LOG PROFILE STYLES
The timbers cut for a log home can be shaped into one of many profiles. These timbers are stacked horizontally to form the walls of your home. Depending on the log’s profile, these logs can be affixed by any number of spikes, nails or fasteners on the market (your log manufacturer’s construction manual can guide this process). The logs can also be “tongue & groove” where cuts to a log’s bottom edge fit into a protrusion created in a lower log’s top. Some log home manufacturers also use chinking between horizontal logs. Depending upon the manufacturer’s system, chinking can be either the functional or cosmetic: The different profiles can include:

### 1. Square & Rectangular Log

Logs are cut with four square corners. These logs can be uniform in width and height or rectangular.

### 2. Round Log

Logs are cut circular, with no angles or corners. Round log homes often use thru-bolts for support. Logs cut circular on each end but with a flat top and bottom are often called “round/round” or “double round” logs.

### 3. Swedish Cope

Logs are cut circular, with a crescent removed from the bottom of the log, so that each log can stack atop another.

### 4. D-Log

Logs are cut with one round side and one flat side for consumers who desire one side of a log wall to have a flat surface while the other retains a rounded edge.

### 5. Handcrafted

Timbers are debarked by hand and each log retains its natural shape. Handcrafted log homes are built so each log is in a precise location in the home, allowing for maximum stability while retaining a rustic look.

## CORNER STYLES

### 1. Interlocking Corners

Wood is cut from the four sides of a log, recessing an area to lock into the intersecting log and hold both logs rigidly in place in all directions. (Similar to the Lincoln Logs you played with as a child.)

### 2. Saddle-Notch Corners

Used with the Swedish cope profile, an additional crescent is cut from each log to allow logs from the opposing wall to lock into place at the corner.

### 3. Dove Tail

A log’s end is cut to produce a fan-shaped wedge. As the logs are stacked, the ends of one wall’s logs lock into the perpendicular logs.

### 4. Butt & Pass

One log stops where it meets a perpendicular log, which extends past the corner of the home.

### 5. Corner Post

A vertical post at each corner has a mortise along its length into which the logs lock.

The next video shows some of the plans and you can see in the background a number of the options the builder has.

https://youtu.be/tBQBgYf1MHg   (time - 9:26)

We’ll take a time lapse look at a similar build from this builder later. We’ve looked at wood and styles of construction lets take a look at some of the plans that are found.

Regardless of your idea of a cabin is this:

Small Log Cabin Building Time-Lapse (Pictures)  https://youtu.be/cpj_IP_DZy8 (Time 6:47)

Or this:

Building a House Time Lapse | Home Construction Start to Finish!  https://youtu.be/_rvoIFiSZmc Time 4:14)

There’s a plan and style for everyone.

Construction

Be it the cabin in the woods or a full-time residence there are tools that are standard with log built home and stick built homes.  For Log building home a list of some more specialized tools are listed below.  This list was taken from a log home building seminar where you learn to build a log home; the tools are still universal in their use for log homes.

LOG BUILDING TOOLS

1)    Draw knife – Barr or Timber Tool drawknives are recommended (9” or bigger)

2)    (Optional) Chisel – mortise or framing chisel (1”, 1 ½” or 2”)

3)    Mallet

4)    Scriber – There are a few different options on Schroeder Log Home Supply, but we recommend the Robert Chambers scriber.

5) (Optional) Log cleats (two sets)

6)    4 or 5-inch angle grinder with separate rubber/plastic sanding backer and #24 or #36 grit discs

7)    Chainsaw and accompanying tools:  scrench (screwdriver-wrench combo), files. Saw should have 16” or 18” bar with .325 pitch. For log building the chain should be chipper or semi-chisel with rounded profile and should be an official, green-linked safety chain

8)    2 gallons of mixed gas for the chainsaw and 1 gallon of bar and chain oil

9)    (Optional) Peavey with a 4- or 5-foot handle

10) Utility knife

11) Tape measure (25 -50 foot)

12)  Chalk line with black or blue chalk

13)  2’ level

14)  Lumber crayons and pencils

15)  Prybar

16)  100’ extension cord

SAFETY GEAR

1)    Hard hat with face shield

2)    Hearing protection (usually the hardhat will have ear muffs built in)

3)    Safety glasses

4)    First aid kit

5)    Leather gloves

6)    Chainsaw protective chaps

7)    Steel toed boots

Examples in our world:

721 107th St. Roberts, WI 54023

Expired listing 5/8/2014 & 4/15/2015

Listed 1,250,000

Log home nestled on 53.96 acres & 3000' lake frontage. Private road to 2BR/2BA home surrounded by wildlife. Bunk House above detached garage. One-of-a-kind retreat!

46696 Otter Shores Road, Candor Twp, MN 56587

Active 11/29/2022 list

$975,000

If you like the outdoors and privacy, this is the spot for you! This property features over 800 feet of lake shore on Otter Lake and over 16 acres of woods. Otter Lake has excellent pan fishing, as well as bass and northern. Otter is a private lake with no public access and minimal owners on the lake, with the opportunity to hunt deer or turkey without leaving your property. Otter Lake also has good water clarity, so you can swim, relax, and enjoy listening to the loons. This property is a beautiful real log home with an Open floor plan, a large loft area, and a walkout basement. Home has a large deck with a wonderful view of the lake. You’ll love this 3-bedroom, 3-bath home with additional square footage to finish in the lower level. This would make a great retreat to enjoy the outdoors and relax!